|  | Simulation of knee_structure Date: Monday, March 14, 2016 Designer: Jorge Rodriguez Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh Information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 13. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: knee_structure Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude1; Solid Body; Mass:0.0203042 kg Volume:1.26901e-005 m^3 Density:1600 kg/m^3 Weight:0.198982 N; D:\Drive\Projects\Legs\src\legs_description\meshes\Iteration 1\structure\knee_structure.SLDPRT Mar 13 20:21:27 2016.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; Off. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (D:\Drive\Projects\Legs\src\legs_description\meshes\Iteration 1\structure).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; Toray T700. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Tensile strength:; 4e+008 N/m^2. Compressive strength:; 3.5e+008 N/m^2. Elastic modulus:; 1.9e+010 N/m^2. Poisson's ratio:; 0.14. Mass density:; 1600 kg/m^3. Shear modulus:; 4.6e+009 N/m^2.; SolidBody 1(Boss-Extrude1)(knee_structure). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.0268427; 0.0375519; -293.247; 293.247. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Reference:; Axis1. Type:; Apply force. Values:; ---, ---, -293.3 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 1.59686 mm. Minimum element size; 1.59686 mm. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 63496. Total Elements; 31816. Maximum Aspect Ratio; 7.2328. % of elements with Aspect Ratio < 3; 99.1. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:04. Computer name:; JORGE-WINDOWS. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 1 component(s). Units:; mm. Size:; 1.59686. Ratio:; 1.5..

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0268427; 0.0375519; -293.247; 293.247. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.7183e+006 N/m^2 Node: 38677; 2.45927e+008 N/m^2 Node: 4900. knee_structure-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 mm Node: 42; 18.6053 mm Node: 19. knee_structure-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 7.96386e-005 Element: 28451; 0.00900996 Element: 23735. knee_structure-Static 1-Strain-Strain1. Image-1.

# Conclusion